Subnet IP Address Calculation

Subnetting is the process of dividing a large IP network into smaller, manageable subnetworks (subnets). It improves network efficiency, security, and IP address utilization.

Key Terms

| Term | Description |
| --- | --- |
| IP Address | A unique 32-bit address assigned to a device. |
| Subnet Mask | Identifies the network and host portions of an IP address. |
| Network Address | The first IP address in a subnet. |
| Broadcast Address | The last IP address in a subnet. |
| Host Address | Addresses assigned to devices between the network and broadcast addresses. |
| CIDR Notation | Represents the subnet mask using a prefix (e.g., /24). |

IPv4 Address Structure

An IPv4 address consists of 32 bits, divided into four octets.

Example:

IP Address: 192.168.10.25

Binary:

192    168      10      25

11000000.10101000.00001010.00011001

Common Subnet Masks

| CIDR | Subnet Mask | Number of Hosts |
| --- | --- | --- |
| /24 | 255.255.255.0 | 254 |
| /25 | 255.255.255.128 | 126 |
| /26 | 255.255.255.192 | 62 |
| /27 | 255.255.255.224 | 30 |
| /28 | 255.255.255.240 | 14 |
| /29 | 255.255.255.248 | 6 |
| /30 | 255.255.255.252 | 2 |

Formula:

- Total addresses = 232-CIDR

- Usable hosts = 232-CIDR-2

(Except for special cases such as /31 and /32.)

Steps for Subnet Calculation

Step 1: Identify the IP Address and CIDR

Example:

IP Address = 192.168.10.75/26

Step 2: Determine the Subnet Mask

/26 = 255.255.255.192

Step 3: Find the Block Size

Block Size =

256 − 192 = 64

Subnet ranges:

0–63

64–127

128–191

192–255

The IP 75 falls in 64–127.

Step 4: Determine the Addresses

Network Address

192.168.10.64

Broadcast Address

192.168.10.127

Usable Host Range

192.168.10.65

to

192.168.10.126

Total addresses

64

Usable hosts

62

Example 1

Given

IP = 172.16.5.130/25

Subnet Mask

255.255.255.128

Block Size

128

Ranges

0–127

128–255

Since 130 lies in 128–255,

Network Address

172.16.5.128

Broadcast Address

172.16.5.255

Host Range

172.16.5.129

to

172.16.5.254

Example 2

Given

IP = 10.10.20.140/28

Subnet Mask

255.255.255.240

Block Size

16

Subnet ranges:

0–15

16–31

32–47

48–63

64–79

80–95

96–111

112–127

128–143

144–159

...

Since 140 falls in 128–143,

Network Address

10.10.20.128

Broadcast Address

10.10.20.143

Host Range

10.10.20.129

to

10.10.20.142

Usable Hosts

14

Shortcut Method

- Write the subnet mask.

- Find the interesting octet (the first octet that is not 255).

- Calculate:

Block Size = 256 − subnet mask value

- List multiples of the block size.

- Locate the subnet containing the given IP.

- Determine:

  - Network Address

  - Broadcast Address

  - Host Range

Practice Problems

Problem 1

IP = 192.168.1.130/25

Answer

- Network: 192.168.1.128

- Broadcast: 192.168.1.255

- Hosts: 192.168.1.129 – 192.168.1.254

Problem 2

IP = 172.20.50.200/27

Answer

- Block Size = 32

- Network = 172.20.50.192

- Broadcast = 172.20.50.223

- Host Range = 172.20.50.193 – 172.20.50.222

Problem 3

IP = 10.0.5.70/29

Answer

- Block Size = 8

- Network = 10.0.5.64

- Broadcast = 10.0.5.71

- Host Range = 10.0.5.65 – 10.0.5.70

Quick Reference Table

| CIDR | Subnet Mask | Block Size | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 255.255.255.0 | 256 | 254 |
| /25 | 255.255.255.128 | 128 | 126 |
| /26 | 255.255.255.192 | 64 | 62 |
| /27 | 255.255.255.224 | 32 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /29 | 255.255.255.248 | 8 | 6 |
| /30 | 255.255.255.252 | 4 | 2 |

Summary

To calculate subnet information:

- Identify the IP address and CIDR prefix.

- Convert the CIDR to its subnet mask.

- Compute the block size: 256 − subnet mask value (in the interesting octet).

- Find the subnet range containing the IP address.

- Determine the network address, broadcast address, and usable host range.

This method is fast, reliable, and commonly used in networking exams such as CCNA as well as in practical network administration.